MSR 200 Series Programming Documentation for ProClerk to work with Keyboard Simulator Magnetic Card Reader.

Perform all programming at a dos command with an AT Keyboard

Hook between AT Keyboard and computer.

Turn power off to computer.

SW4 – Off

SW5 – On

SW8 – Off

Turn power on to computer.

When computer says starting Windows hold down Shift and press F5 to stop and go to command line.

Type UR and press enter to reset card reader to default.

Type UPE and press enter to set prefix and suffix on each track.

Type UEE and press space bar to add space between tracks – Do not press enter!

Turn power off to computer.

SW8 – On

Turn power back on to computer.