MSR 210 Series Programming Documentation for ProClerk to work with Keyboard Simulator Magnetic Card Reader.

- Power off PC, connect the reader with PC, connect AT Keyboard with PS2 adapter to reader and then power on.

- All programming must be done with external keyboard if you have a laptop.

- Execute text editor program such as Microsoft Word, Notepad, DOS PE2, Edit, …etc.

- Press and hold [Left-Ctrl], [Left-Alt], and [F10] keys in order to enter Configuration Mode.

- Once you enter the configuration mode, the green LED is always blinking.  You should then se the following menu:

**** configuration mode ****

1:set interface

2:set magnetic string

3:set string editing

4:set buzzer

5:reset to default

6:show status

7:set keyboard config

0:exit setup mode

Set carriage return after all tracks.

Choose 2 – SET MAGNETIC STRING – You should see the following options:

1:set enable tracks

2:set require tracks

3:carriage return

4:transmit ss/es

5:caps lock

6:track output order

7:change carriage return code

8:output cr if read nothing

0:back

Choose 3 – CARRIAGE RETURN – You should see the following options:

*** carriage return ***

1:don’t send carriage return

2:after all tracks

3:after each track

4:after all data

0:back

reader status each after all tracks

Choose 2 – AFTER ALL TRACKS

Choose 0 – until you get back to the original configuration mode menu

Program space character separator between tracks

**** configuration mode ****

1:set interface

2:set magnetic string

3:set string editing

4:set buzzer

5:reset to default

6:show status

7:set keyboard config

0:exit setup mode

Choose 3 – SET STRING EDITING – You should see the following options:

1:set prefix

2:set suffix

3:set magnetic stripe preamble

4:set magnetic stripe postamble

5:block output order

6:send control

7:field output order

8:exp date format

9:exp date separator

0:back

Choose 2 – SET SUFFIX – You should see the following options:

1:track 1

2:track 2

3:track 3

0:back

Choose 1 – TRACK 1 – You should see the following options:

*** set track 1 suffix ***

1:enable

2:disable

0:back

Reader status = disable

Choose 1 – ENABLE – You should see the following options:

enter suffix (6 chars max) then press ‘ when finished

important! [caps] key is not allowed

PRESS THE SPACE BAR 1 TIME AND TYPE THE ‘ KEY

You should automatically be returned back to the following menu:

1:track 1

2:track 2

3:track 3

0:back

Choose 2 – TRACK 2 – You should see the following options:

*** set track 2 suffix ***

1:enable

2:disable

0:back

Reader status = disable

Choose 1 – ENABLE – You should see the following options:

enter suffix (6 chars max) then press ‘ when finished

important! [caps] key is not allowed

PRESS THE SPACE BAR 1 TIME AND TYPE THE ‘ KEY

You should automatically be returned back to the following menu:

1:track 1

2:track 2

3:track 3

0:back

Choose 3 – TRACK 3 – You should see the following options:

*** set track 3 suffix ***

1:enable

2:disable

0:back

Reader status = disable

Choose 1 – ENABLE – You should see the following options:

enter suffix (6 chars max) then press ‘ when finished

important! [caps] key is not allowed

PRESS THE SPACE BAR 1 TIME AND TYPE THE ‘ KEY

CHECK THE STATUS

Select 0 until you are returned back to the main configuration menu:

**** configuration menu ****

1:set interface

2:set magnetic string

3:set string editing

4:set buzzer

5:reset to default

6:show status

7:set keyboard config

0:exit setup mode

Select 6 – SHOW STATUS

You should see the following settings.  If not press 0 to return to the main configuration menu above and choose option 5 to reset to default and then start over from the start of these instructions.

keyboard country=u.s.

transmit speed=120 char/sec

enable tracks=track 1,2,3

require tracks=any track

carriage return=after all tracks

ss/es send=enable

caps lock=enable

buzzer=enable

data output=1,2,3

keyboard = standard config

press 0 to back

EXITING SETUP

Press 0 until the green led stops flashing